Annex C

Technical Proposal

| Name of Participant: | [Insert Name of Participant] | Date: |  |
| --- | --- | --- | --- |
| CFB reference: | No. CFB/MDA/EQ3/ATV/18/2026 |  |  |

Where a descriptive response is requested, failure to provide the same will be viewed as non-responsive.

The bidder's offer should be organized to follow this format of the Technical Proposal. The participant must describe the exact type of each offered item and provide relevant product information or visualization for each item. The quality of the goods offered may exceed the minimum technical requirements of this CFB.

| No | MINIMUM REQUIREMENTS | Technical specification | OFFER 1 | OFFER 2 | OFFER 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Brand / Model / PN / Origin | Brand / Model / PN / Origin | Brand / Model / PN / Origin |
| 1 | Field mobility – all-terrain vehicle (ATV) | All-terrain vehicle (ATV) platform to improve movement, access and light transport of personnel and light loads in difficult terrain or degraded operating environments; single-seat configuration with selectable two-wheel drive / on-demand four-wheel drive (AWD); four-stroke, liquid-cooled, single-cylinder petrol engine with electronic fuel injection (EFI), displacement ≥ 500 cm³, rated power ≥ 30 kW (≥ 40 HP); automatic (CVT/PVT) transmission with high/low range, reverse and park; electric power steering (EPS); hydraulic disc brakes front and rear with parking brake; independent front and rear suspension, minimum ground clearance 27 cm; all-terrain tyres suitable for off-road use; payload capacity ≥ 200 kg; towing capacity ≥ 550 kg; combined front/rear cargo rack capacity ≥ 110 kg; fuel tank minimum 15 L; integrated front cargo box/dry storage; standard hitch receiver; 12V DC accessory outlet; LED lighting; electric winch, minimum 1,500 kg (3,300 lb) pulling capacity; digital instrument cluster (speedometer, odometer, fuel gauge, hour meter); CE marking or equivalent recognized conformity marking; complete with tool kit and owner's/operator manual/documentation in English. |  |  |  |

We attach the following:

a) Technical information (if necessary) – ……. (number of pages);

Authorized signature: ___________________________

(Name of authorized signatory)

Functional Title: ___________________